EXPERIMENT – 2.2

BLINK AN EXTERNAL LED USING BUTTON ON DEV BOARD/NODE

What will you learn from this module:

Blink an external LED using button on Development Board/Node.

Requirements:

- nRF connect desktop software.

- nRF Command line tools.

- Visual studio code.

- USB cable.

- nRF52832 Development Board/Node.

- LED’s.

Prerequisites:

- Basic knowledge of C/C++

- Basic knowledge of communication protocol.

- Basic project setup.

Setup and Configuration:

- Open VS Code and go to Browse samples [1] and search Button [2].

- Click on Create new build configuration [1]. Here you can change the board version, if you are using nRF52832, then select nrf52dk_nrf52832 [2] or you can change from dropdown menu for another version like nRF52833 etc.

- Click on the Configuration and select prj.config [3] from dropdown menu and then click on the Build Configuration [4].

- Go to source file [2],click source file > click on Application > click on src [3] > click on main.c [4] .

- By clicking on main.c file and you will see the code on your screen.

- Click on the Config files [1] > Devicetree > .dts file [2].

- Change the gpio pin no. [3] with the pin that is used by the external led on the nRF board. (for example, if led’s +ve terminal is connected to P0.25 on the development board & -ve terminal to GND, then change the gpio pin no. to 25 in the dts as shown in the figure.

- Run the build configuration again.

- Then flash the code in nRF dev kit.

- Click on Build [1] configuration again and check the CONNECTED DEVICES [2].

- If device id is visible, then Flash the code in Development Kit.

- If flashed successfully message is displayed on serial terminal, then flash process is complete.

- PIN CONFIGURATION

Board Pins -> LED Pins

P0.25V -> +ve terminal

GND -> -ve terminal

- OUTPUT

- nRF52832 board after press the button.

- nRF52832 board before press the button.

Press the button and LED will glow

- WITH THE HELP OF NODE

- For Node programing remove the jumper J2 from the development board.

- Now flash the code with the help of nRF52832 development board as shown below in the figure.

Board Pins -> NODE Pins

VDD(3.3V) -> VDD

GND -> GND

CLK -> CLK

DIO -> DIO

Remove the jumper J2 from here.

- There is another way of flashing the code with the help of Node Programmer as shown in the picture below.

- NODE after program.

- NODE with connection.

- NODE without connection.

- PIN CONFIGURATION

LED Pins -> NODE Pins

+ve  -> SCL

-ve -> GND

- OUTPUT

- NODE after flash the code.

- NODE before flash the code.

Press the button and LED will glow